PRODUCT FORMULATION

I, the undersigned Martin Stevens, Technical Director, Potter + Moore Innovations Ltd hereby certify that the following cosmetic product is manufactured in our factory located at 1210 Lincoln Road, Werrington, Peterborough, PE4 6ND in accordance with the Good Manufacturing Practices Guidelines and complies with the Cosmetic Product (Safety) Regulations 2008 "as amended"

and EC1223 - 2009

Product: Hair Design Studio Just No Frizz Frizz Fighting Shampoo

FORMULATION                                                                MCR12561

| INCI | CAS No. | EINECS No. | Function | % |
| --- | --- | --- | --- | --- |
| Aqua (Water) | 7732-18-5 | 231-791-2 | Solvent | 84.19134000 |
| Sodium Laureth Sulfate | 3088-31-1 | 221-416-0 | Surfactant | 7.65000000 |
| Sodium Chloride | 7647-14-5 | 231-598-3 | Viscosity Control | 2.95050000 |
| Cocamidopropyl Betaine | 61789-40-0 | 263-058-8 | Foam booster, Surfactant | 2.63250000 |
| PEG-7 Glyceryl Cocoate | 66105-29-1 | None | Emulsifier, Surfactant | 1.00000000 |
| Parfum (Fragrance) | Mixture | Mixture | Fragrance | 0.44750000 |
| Glycerin | 56-81-5 | 200-289-5 | Humectant, Moisturiser | 0.25000000 |
| Hydroxypropyltrimonium Hydrolyzed Corn Starch | None | None | Antistatic agent, Film former, Hair conditioning | 0.22400000 |
| Glycol Cetearate | 111-60-4 | 203-886-9 | Emollient, Emulsifier, Emulsion stabiliser | 0.12750000 |
| Panthenol | 81-13-0 | 201-327-3 | Antistatic agent, Hair conditioning | 0.11250000 |
| Polyquaternium-7 | 26590-05-6 | None | Antistatic agent, Film Former | 0.10800000 |
| Hydroxypropyl Guar Hydroxypropyltrimonium Chloride | 71329-50-5 | None | Antistatic agent, Hair conditioning | 0.10000000 |
| Dimethicone PEG-8 Meadowfoamate | None | None | Emollient, Skin conditioning, Hair conditioning | 0.05000000 |
| Disodium EDTA | 139-33-3 | 205-358-3 | Chelating | 0.05000000 |
| Tetrasodium EDTA | 64-02-8 | 200-573-9 | Chelating agent | 0.01500000 |
| Magnesium Nitrate | 10377-60-3 | 233-826-7 | Skin Conditioning | 0.01295000 |
| Citric Acid | 77-92-9 | 201-069-1 | pH adjustment | 0.01000000 |
| Phenoxyethanol | 122-99-6 | 204-589-7 | Preservative, Antimicrobial | 0.00640000 |
| Sodium Benzoate | 532-32-1111 | 208-534-8 | Preservative, Antimicrobial | 0.00480000 |
| Sodium Hydroxide | 1310-73-2 | 215-185-5 | pH adjustment | 0.00320000 |
| Methylchloroisothiazolinone | 26172-55-4 | 247-500-7 | Preservative, Antimicrobial | 0.00066000 |
| Magnesium Chloride | 7786-30-3 | 232-094-6 | Viscosity Modifier | 0.00044000 |
| Methylisothiazolinone | 2682-20-4 | 220-239-6 | Preservative, Antimicrobial | 0.00022000 |
| Linalool | 78-70-6 | 201-134-4 | Fragrance Component | 0.01699000 |
| Butylphenyl Methylpropional | 80-54-6 | 201-289-8 | Fragrance Component | 0.01325000 |
| Limonene | 138-86-3 | 205-341-0/931-893-3 | Fragrance Component | 0.01225000 |
| Benzyl Salicylate | 118-58-1 | 204-262-9 | Fragrance Component | 0.01000000 |

MARTIN STEVENS TECHNICAL DIRECTOR For and on behalf of

Potter+Moore Innovations Ltd

1210 Lincoln Road  Peterborough  England  PE4 6ND (Registered Office) Tel: +44 (0) 1733 281000   Fax: +44 (0) 1733 281028

Registered in England No : 4645119

100.00000000